EXPERIMENT NO -10

## Procedure to Find Soil Moisture Content Using Arduino IDE in Tinkercad

### Aim

To measure the soil moisture content using an Arduino Uno and a soil moisture sensor, simulated in Tinkercad.

### Components Required

- Arduino Uno

- Soil Moisture Sensor

- Jumper wires

- Computer with Arduino IDE / Tinkercad Circuits

### Circuit Connections

Connect the soil moisture sensor to the Arduino Uno as follows:

| Soil Moisture Sensor | Arduino Uno |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SIG/AO | A0 |

### Procedure

- Open Tinkercad and sign in.

- Go to Circuits → Create new Circuit.

- Place an Arduino Uno R3 in the workspace.

- Add a Soil Moisture Sensor to the circuit.

- Connect:

  - Sensor VCC → Arduino 5V

  - Sensor GND → Arduino GND

  - Sensor SIG → Arduino A0

- Click Code and select Text mode.

- Enter the following Arduino program:

int moistureValue;

float moisture_percentage;

void setup()

{

Serial.begin(9600);

}

void loop()

{

moistureValue = analogRead(A0);

moisture_percentage = ((moistureValue / 539.00) * 100);

Serial.print("\nMoisture value: ");

Serial.print(moisture_percentage);

Serial.print("%");

delay(1000);

}

- Click Start Simulation.

- Open the Serial Monitor in Tinkercad.

- Observe the moisture percentage displayed in the Serial Monitor.

- Change the sensor's simulated moisture/environment setting in Tinkercad and observe how the moisture value and percentage change.

- Record the readings for different simulated soil conditions.

### Result

The soil moisture content is measured using the analog output of the soil moisture sensor. The calculated moisture percentage is displayed on the Serial Monitor of the Arduino simulation.